# CAN-ETH Gateway Python SDK

Python SDK for the STM32H755 CAN-to-Ethernet gateway.  Supports all four

transport paths: UDP CAN inject (TX), CLOG v2 stream (RX), status heartbeat,

and the HTTP REST API.

## Quick Start

cd validation/

pip install -r requirements.txt

# Run the GUI demo

python demo/main.py

# Or use the SDK directly from a script

python - <<'EOF'

from sdk import GatewayClient

with GatewayClient("192.168.1.100") as gw:

gw.clog.on_frame = lambda f: print(f)

# Send a raw CAN frame on FDCAN1

gw.send_can(0x123, b"\x01\x02\x03\x04", extended=False)

# Send a J1939 broadcast

gw.send_j1939(pgn=0xFEF1, sa=0x00,

data=b"\xFF\xFF\x00\x00\xFF\xFF\xFF\xFF")

import time; time.sleep(5)   # let CLOG frames arrive

EOF

## Package Layout

validation/

├── requirements.txt          # Python dependencies

├── sdk/

│   ├── __init__.py           # exports GatewayClient

│   ├── client.py             # GatewayClient — unified entry point

│   ├── protocols/

│   │   ├── can_inject.py     # CAN TX  (UDP port 4000)

│   │   ├── clog.py           # CAN RX  (UDP port 47808, CLOG v2)

│   │   ├── status.py         # Heartbeat (UDP multicast 239.1.2.3:7898)

│   │   └── j1939.py          # J1939 PGN utilities

│   └── http/

│       └── api.py            # HTTP REST API wrapper (port 80)

├── demo/

│   └── main.py               # PyQt5 GUI application

└── docs/

└── README.md             # this file

## GatewayClient

GatewayClient is the single object that owns all transports.

from sdk import GatewayClient

gw = GatewayClient(

host="192.168.1.100",    # gateway IP

inject_port=4000,        # UDP CAN inject port (default)

clog_port=47808,         # CLOG receive port (default)

status_port=7898,        # status heartbeat port (default)

http_port=80,            # HTTP API port (default)

listen_ip="0.0.0.0",     # local bind address

)

gw.open()   # or use as context manager: with GatewayClient(...) as gw:

...

gw.close()

### CAN TX

# Standard 11-bit frame

gw.send_can(0x123, b"\xDE\xAD\xBE\xEF")

# Extended 29-bit frame on FDCAN2

gw.send_can(0x18FF0000, b"\x01\x02", extended=True, channel=1)

### J1939 TX

# Broadcast engine speed PGN on FDCAN1

gw.send_j1939(

pgn=0xFEF1,          # Cruise Control / Vehicle Speed

sa=0x00,             # Source Address

data=b"\xFF\xFF\x00\x00\xFF\xFF\xFF\xFF",

da=0xFF,             # 0xFF = global broadcast

priority=6,

channel=0,

)

# Or build a J1939Frame and pass it directly

from sdk.protocols.j1939 import J1939Frame

frame = J1939Frame(pgn=0xFECA, sa=0x01, data=b"\x00" * 8)

gw.send_j1939_frame(frame)

### CAN RX (CLOG)

CLOG frames arrive asynchronously on a background thread.

from sdk.protocols.clog import CLOGType

def on_frame(f):

if f.msg_type == CLOGType.RAW_CAN:

print(f"CAN  id=0x{f.can_id:08X} data={f.data.hex()}")

elif f.msg_type == CLOGType.J1939:

print(f"J1939 PGN=0x{f.pgn:05X} SA=0x{f.sa:02X} data={f.data.hex()}")

elif f.msg_type == CLOGType.STATUS:

print(f"Gateway uptime={f.uptime_sec}s ch1_rx={f.ch1_rx}")

gw.clog.on_frame = on_frame

# Or poll the internal buffer

import time; time.sleep(1)

frames = gw.clog.get_frames(clear=True)

### Status Heartbeat

Sent by the gateway every 1 second to UDP multicast 239.1.2.3:7898.

gw.status.on_frame = lambda s: print(f"IP={s.ip_addr} uptime={s.uptime_s}s")

# Or read the most recent frame directly

s = gw.last_status

if s:

print(f"Free heap: {s.free_heap} bytes")

### HTTP REST API

# Device info

info = gw.api.get_info()

# J1939 telemetry

tel = gw.api.get_telemetry()

# Active diagnostic trouble codes (DM1)

dtc = gw.api.get_dtc()

# Send a CAN frame via HTTP (alternative to UDP inject)

gw.api.send_can(0x123, b"\x01\x02\x03", extended=False)

# Configure Radxa WiFi

gw.api.set_radxa_wifi("MySSID", "MyPassword")

# Download configuration blob

blob = gw.api.get_config_blob()

## Protocol Reference

### CAN Inject (UDP port 4000)

| Offset | Size | Field | Description |
| --- | --- | --- | --- |
| 0 | 1 | Magic | 0xCA |
| 1 | 1 | Channel | 0 = FDCAN1, 1 = FDCAN2 |
| 2 | 1 | Flags | see flags table below |
| 3 | 1 | DataLen | payload length 0–64 |
| 4–7 | 4 | CAN ID | uint32 little-endian |
| 8+ | 0–64 | Payload | CAN data bytes |

Flags:

| Bit | Name | Meaning |
| --- | --- | --- |
| 0 | EXT | 29-bit extended identifier |
| 1 | J1939 | J1939 routing (informational) |
| 2 | FD | CAN FD frame |
| 3 | BRS | Bit-rate switch (FD only) |

### CLOG v2 (UDP port 47808, big-endian)

Common Header (28 bytes):

| Offset | Size | Field | Description |
| --- | --- | --- | --- |
| 0–3 | 4 | Magic | "CLOG" (0x43 0x4C 0x4F 0x47) |
| 4 | 1 | Version | 2 |
| 5 | 1 | Msg type | 0=STATUS, 1=RAW_CAN, 2=J1939 |
| 6 | 1 | Channel ID | logging channel 0–255 |
| 7 | 1 | Flags | FD/BRS/ESI/EXT/RTR |
| 8–11 | 4 | Sequence | per-channel counter (BE) |
| 12–15 | 4 | TS seconds | PTP TAI (BE) |
| 16–19 | 4 | TS nanosec | PTP nanoseconds (BE) |
| 20–23 | 4 | CAN ID | 29-bit ID (BE); 0 for STATUS |
| 24 | 1 | DLC | CAN DLC 0–15 |
| 25–27 | 3 | Reserved | 0 |

Type 0 STATUS payload (40 bytes @ offset 28):

| Offset | Field | Description |
| --- | --- | --- |
| 28 | status_flags | ETH_UP / CAN1_ACTIVE / CAN2_ACTIVE / PTP_LOCKED |
| 29 | can1_state | OK/WARNING/PASSIVE/BUS_OFF |
| 30 | can2_state | same |
| 31 | fw_major |  |
| 32 | fw_minor |  |
| 36–39 | uptime_sec | BE uint32 |
| 40–67 | counters | ch1/ch2 rx/tx/error, dropped (BE uint32) |

Type 2 J1939 payload (8-byte routing header @ offset 28 + data):

| Offset | Field | Description |
| --- | --- | --- |
| 28 | priority | J1939 priority 0–7 |
| 29 | sa | Source Address |
| 30 | da | Destination Address (0xFF = broadcast) |
| 31 | reserved | 0 |
| 32–35 | pgn | 18-bit PGN in lower bits (BE uint32) |
| 36+ | data | CAN payload, length = DLC_to_len(dlc) |

### Status Heartbeat (UDP multicast 239.1.2.3:7898, 20 bytes)

| Offset | Size | Endian | Field |
| --- | --- | --- | --- |
| 0 | 1 | — | msg_type (0x01) |
| 1 | 1 | — | dev_id |
| 2 | 1 | — | proto_ver |
| 3 | 1 | — | flags |
| 4–7 | 4 | LE | uptime_s |
| 8–11 | 4 | BE/NBO | ip_addr |
| 12–15 | 4 | LE | free_heap |
| 16–17 | 2 | LE | task_count |
| 18–19 | 2 | — | reserved |

## Running Tests

cd validation/

# Install deps

pip install -r requirements.txt

# Quick smoke test (no hardware needed)

python -c "

from sdk.protocols.j1939 import pgn_from_can_id, can_id_from_j1939

# Round-trip J1939 ID encode/decode

pgn, sa, da, prio = pgn_from_can_id(0x18FEF100)

assert pgn == 0xFEF1, f'got {pgn:#07x}'

assert sa == 0x00

print('J1939 codec OK')

# CLOG parse

from sdk.protocols.clog import _parse

import struct

hdr = struct.pack('>4sBBBBIIIIB3s',

b'CLOG', 2, 1, 0, 0x08, 1, 0, 0, 0x123, 3, b'\\x00\\x00\\x00')

frame = _parse(hdr + b'\\xDE\\xAD\\xBE')

assert frame.can_id == 0x123

assert frame.data == b'\\xDE\\xAD\\xBE'

print('CLOG parse OK')

"